1. Analyze the unfiltered signal in the time domain

2. Analyze the unfiltered signal in the frequency domain

3. Design a suitable filter (order, frequencies, etc.) according to the given criteria.

Given Criteria

Undesired Frequency Range: 172.8-211.2 Hz for the data file: ADC11

This requires us to design a ‘bandstop/notch’ type filter, for which

Stopband Attenuation (Rs) ≤0.2×(Minimum Stopband Amplitude)*

[*This corresponds to a maximum allowable gain of: 10log1015≅-7 dB]

The MATLAB app, Signal Anlaysis, is used to implement the filter designed through the app named Filter Design & Analysis.

Where the filter specifications above are illustrated as;

5. Analyze the filtered signal in the frequency domain

6. Analyze the filtered signal in the time domain

7. Redesign the filter if required.

- By narrowing ωp the filter is noticed to become unstable.

- By widening ωp, the filter yields acceptable results and reduced order, hence reduced cost of implementing.

8. Implement the digital filter in the form of a DIFFERENCE EQUATION in Matlab script language and analyze the filtered signal.

The MATLAB function buttord gives a suitable order as n=9 and a normalised cutoff frequency range ωn[0.3426...   0.4255...] for the following parameters:

| Parameter | Value(s) |
| --- | --- |
| (Normalised) Passband corner frequency (ωp) | 0.3420-0.4260 rad sample-1 |
| (Normalised) Stopband corner frequencies (ωs) | 0.3456-0.4224 rad sample-1 |
| Maximum Passband Ripple (Rp) | 3 dB |
| Minimum Stopband Attenuation (Rs) | -10log1015 dB |

These are then used with the function, butter, to generate coefficients for the difference equation.

[N.B. The same passband frequencies resulted in instability, thus we widened the passband]

Comparing plots in the result of step 6 and 8. These are close enough.

Analysis code and implemented digital filter code:

% The signal is sampled at 1000 Hz

FS = 1000 ; % Sampling frequency

% According to Nyquist theorm we can only have frequencies upto 500 Hz

%   in this signal. The function butter take normalised frequency.

FN = FS/2; % Nyquist frequency

figure(1)

plot (t,x) ; grid

title ('Time Plot of Unfiltered Signal')

xlabel('Time (sec)')

ylabel('Magnitude')

% Plot first half of DFT (normalised frequency)

X_mags = abs(fft(x))/FS;

num_bins = length(X_mags);

figure(2)

plot([0:1/(num_bins/2 -1):1]*FN, X_mags(1:(num_bins-1)/2))

title ('Frequency Plot of Unfiltered signal')

xlabel('Frequency (Hz)')

ylabel('Magnitude')

format long

x_dfiltered=(ADC11_F.data)';

% Plot first half of DFT (normalised frequency)

X_mags = abs(fft(x_dfiltered))/FS;

num_bins = length(X_mags);

figure(4)

plot([0:1/(num_bins/2 -1):1]*FN, X_mags(1:(num_bins-1)/2))

title ('Frequency Plot of Filtered signal')

xlabel('Frequency (Hz)')

ylabel('Magnitude')

figure(5)

plot (t,x_dfiltered) ; grid

title ('Time Plot of Filtered Signal')

xlabel('Time (sec)')

ylabel('Magnitude')

% Design butterworth filter

Wp=[171 213]./FN%Passband (normalised) corner frequencies (rad/sample)

Ws=[172.8 211.2]./FN%Stopband (normalised) corner frequencies (rad/sample)

Rp=3%Maximum Passband Ripple (dB)

Rs=-10*log10(1/5)%Minimum Attenuation of the Undesired Frequency range (dB)

[n,Wn] = buttord(Wp,Ws,Rp,Rs) %n: Order of the Filter    Wn: Normalized cutoff frequencies

[b a] = butter(n, Wn, 'stop');

%plot the frequency response (normalised frequency)

H = freqz(b,a, floor(num_bins/2));

figure(3);

plot([0:1/(num_bins/2 -1):1]*FN, abs(H),'r');grid

title ('Frequency Response of Difference Equation form Filter')

xlabel('Frequency (Hz)')

ylabel('Magnitude')

x_DEfiltered=filter(b, a, x);

% Plot first half of DFT (normalised frequency)

X_mags = abs(fft(x_DEfiltered))/FS;

num_bins = length(X_mags);

figure(6)

plot([0:1/(num_bins/2 -1):1]*FN, X_mags(1:(num_bins-1)/2))

title ('Frequency Plot of Difference Equation form Filtered signal')

xlabel('Frequency (Hz)')

ylabel('Magnitude')

figure(7)

plot (t,x_DEfiltered) ; grid

title ('Time Plot of Difference Equation form Filtered Signal')

xlabel('Time (sec)')

ylabel('Magnitude')